DIAGNOSIS

| 1. | Turn ignition switch to ON. |
| --- | --- |

| 2. | The Air Bag Warning Light should illuminate. If it does not, refer to Air Bag Warning Light service. |
| --- | --- |

| 3. | The Air Bag Warning Light should go out after approximately 5 seconds. If it stays out, the system is OK. |
| --- | --- |

| 4. | If the Air Bag Warning Light remains illuminated, there is a problem in the Air Bag-Supplemental Restraint System (SRS). |
| --- | --- |

| 5. | Turn the ignition switch to LOCK. |
| --- | --- |

| 6. | Locate the data link connector (DLC) in the left side of the engine compartment and install the Scan Tool. |
| --- | --- |

| 7. | Turn the ignition switch to ON and record any Air Bag Diagnostic Trouble Codes displayed. |
| --- | --- |

| 8. | Refer to Air Bag Diagnostic Trouble Code Chart for fault descriptions and actions. |
| --- | --- |

AIR BAG DTC CHART

| Air Bag Diagnostic Trouble Code (DTC) Chart |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Diagnostic Trouble Code | Faul Description | Possible Cause | Warning Light Code | Air Bag Module Operational | Corrective Action |
| 00 | Crash Occurred | Crash data stored | 1 | All Yes | Replace Air Bag Diagnostic Unit (ADU) and test system |
| 10 | Low Ignition Power Supply | Ignition Power Supply voltage below 9 volts | 3 | All Yes | Repair or replace battery or harness |
| 11 | High Ignition Power Supply | Accessory Power Supply voltage above 16 volts | 3 | All Yes | Repair or replace battery or harness |
| 31 | ADU Fault | Internal fault | 1 | Possible depending on type of fault | Replace Air Bag Diagnostic Unit (ADU) |
| 37 | ADU Incomplete Connection | Connector Fault | 1 | Possible depending on connection terminals | Reconnect ADU connector completely or replace harness |
| 42 | Warning Light Short to Ground | Warning Light damaged, or circuit shorted to ground | - | All Yes | Repair or replace warning light or harness |
| 44 | Warning Light Short to Battery | Warning Light damaged, or circuit shorted to battery | - | All Yes | Repair or replace warning light or harness |
| 57 | Passenger Air Bag Module Short to Ground | Passenger Air Bag module damaged, or shorted to ground | 7 | Passenger : No Steering wheel, knee : Yes | Replace module or harness |
| 58 | Passenger Air Bag Module Short to Battery | Passenger Air Bag module damaged, or circuit shorted to Battery | 7 | Passenger : No Steering wheel, knee : Yes | Replace module or harness |
| 59 | Passenger Air Bag Module Short | Passenger Air Bag module damaged, or open circuit in harness | 7 | Passenger : No Steering wheel, knee : Yes | Replace module or harness |
| 60 | Passenger Air Bag Module Open | Passenger Air Bag module damaged, or open circuit in harness | 7 | Passenger : No Steering wheel, knee : Yes | Replace module or harness |
| 61 | Steering Wheel Air Bag Module Short to Ground | Steering Wheel Air Bag module damaged, or shorted to ground | 6 | Steering wheel : No Passenger, knee : Yes | Replace module or hanress |
| 63 | Steering Wheel Air Bag Module Open | Steering Wheel Air Bag module damaged, or open circuit in harness | 6 | Steering wheel : No Passenger, knee : Yes | Replace module or harness |
| 65 | Steering Wheel Air Bag Module Short | Steering Wheel Air Bag module damaged, or short circuit in harness | 6 | Steering wheel : No Passenger, knee : Yes | Replace module or harness |
| 67 | Steering Wheel Air Bag Module Short to Battery | Steering Wheel Air Bag module damaged, or circuit shorted to battery | 6 | Steering wheel : No Passenger, knee : Yes | Replace module or harness |
| 78 | ADU Inconsistency | ADU unmaches vehicle installed modules | 2 | Possible depending on ADU type | Replace Air Bag Diagnostic Unit (ADU) |
| 91 | Knee Air Bag Module Short to Ground | Knee Air Bag module damaged, or circuit shorted to ground | 8 | Knee : No Steering wheel, Passenger : Yes | Replace module or harness |
| 93 | Knee Air Bag Module Open | Knee Air Bag module damaged, or open circuit in harness | 8 | Knee : No Steering wheel, Passenger : Yes | Replace module or harness |
| 95 | Knee Air Bag Module Short | Knee Air Bag module damaged, or short circuit in harness | 8 | Knee : No Steering wheel, Passenger : Yes | Replace module or harness |
| 97 | Knee Air Bag Module Short to Battery | Knee Air Bag module damaged, or circuit shorted to battery | 8 | Knee : No Steering wheel, Passenger : Yes | Replace module or harness |

WARNING LIGHT CODE

Warning light will be illuminated for remainder, of ignition ON cycle. It spit of being self-corrected the fault, but will not be illuminated after next ignition ON cycle if the fault has erased by Scan Tool.

| Air bag diagnostic unitconnector |  |  |  |
| --- | --- | --- | --- |
| Terminal | Description | Terminal | Description |
| 1 | - | 12 | - |
| 2 | - | 13 | Passenger air bag module (+) |
| 3 | - | 14 | Air Bag warming light (+) |
| 4 | - | 15 | Driver air bag module (+) |
| 5 | Knee air bag module (+) | 16 | - |
| 6 | - | 17 | - |
| 7 | - | 18 | - |
| 8 | Supply power (IG1) (+) | 19 | - |
| 9 | Communication (K-line) | 20 | Driver air bag module (-) |
| 10 | Knee air bag module (-) | 21 | Ground |
| 11 | - | 22 | Passenger air bag module (-) |

AIR BAG DIAGNOSTIC TROUBLE CODE ERASURE

Disconnecting the Battery will not erase Air Bag Diagnostic Trouble Codes. This procedure can only be performed using the Kia Data Pro-II Tool equipped with the appropriate software.